# Discussion Format

Discussions will be held in accordance to steps 1 and 2 after planned scientific talks.

# Updates

Important updates will be communicated during the discussions, less important updates will be stored at a dedicated page of the MCh wiki in this format.

# Deposition Template(s)

- units:

  - prefilled drop-down menu of standardly used unit

  - take steps to notify users there is a drop-down menu (colors, formatting)

- gas supply:

  - store both gas flow and partial pressure of used gases

- magnetrons:

  - store material system in the same manner as the general material system

  - store composition provided by a producer/supplier

  - targets will be identifiable via a barcode, which needs to be stored as well

- substrate:

  - store ID of a substrate and its supplier

  - heating needs to allow discerning intended and not-intended heating

    - Boolean switch

    - when intended heating, store temperature and temperature rate and also store additional information regarding heating set-up (e.g. power of heating spirals)

- generators:

  - power instead of voltages (RF mode)

  - store current as well as maximum value of a fixed parameter (mostly HPPMS mode)

- general:

  - store "equilibration time" as well as pressure before and after heating

- include more comment-like metadata (if possible)